#### Section VII.

#### Effects of Settleable Solids on Benthos and Shellfish

Q33. Are the proposed rearing units located directly in a Great Lake or a connecting water body of a Great Lake?

Proceed to Section VIII,  Impacts on Breeding Areas, Nurseries and Fish-eating animals.

A2. no

A1. yes

Provide documentation of efficacy, then proceed to Section VIII, Impacts on Breeding Areas, Nurseries and Fish-eating animals.

Q34. Are there waste containment and collection measures in place for excess food, feces, and mortalities?

A1. yes

H20. This poses a hazard to shellfish that are intended for harvest. Operator should develop plans minimize release of settleable solids. If these plans are acceptable, and you are willing to accept risk to the shellfish at issue, proceed to Question 36. If not, consider relocating facility or reducing volume of production.

A2. no

Q35. Is proposed facility within a “zone of influence” (see definition) for harvested shellfish beds? Note endangered benthic organisms are addressed in Recovery and Rehabilitation assessment.

A1. yes

A2. no

Q36. Is water distance from bottom of cages to lake substrate sufficient enough to avoid impeding water flow and causing excessively high  sedimentation of aquacultural waste?

H21. This poses hazard to benthic organisms. Operator should develop plans minimize release of settleable solids. If these plans are acceptable, and you are willing to accept risk to the benthos at issue, proceed to Question 44. If  not, consider facility  relocation or reduce volume of production.

A1. yes

Go to Question 37.

A2. no

Q37. Is water current velocity sufficiently strong enough to disperse aquacultural waste products, maximize water exchange and restore oxygen levels but not so strong as to impede the feeding ability of cultured organisms?

H22. This poses a hazard to benthic organisms. Operator should develop plans to minimize release of settleable solids. If these plans are acceptable, and you are willing to accept risk to the benthos at issue, proceed to Question 38.  If not, consider relocating facility or reducing volume of production.

A2. no

A1. yes

Q38. Will the addition of this facility to other neighboring aquaculture facilities result in adverse cumulative effects on benthic organisms?  In other words, will there be an overlap of settleable solids on benthos if this facility is located at the proposed site?

H23. This poses a significant hazard to benthic organisms. Operator should develop plans to minimize release of settleable solids. If these plans are acceptable, and you are willing to accept risk to the benthos at issue, proceed to Question 39. If not, consider relocating facility or reducing volume of production.

A1. yes

A2. no

Q39. Are there known fouling agents (zebra mussels) in proposed facility area?

H24. This poses a hazard to the culture operation (see supporting text). If willing to accept risk, proceed to  Section VIII,  Impacts on Breeding Areas, Nurseries and Fish-eating animals. If not, consider relocating facility.

A1. yes

A2. no

Proceed to  Section VIII,  Impacts on Breeding Areas, Nurseries and Fish-eating animals.